- Describe Transport Layer Protocols.

- Differentiate connection-oriented and connectionless services.

- Explain UDP user datagram header format.

- Example of UDP hex dump.

- Explain UDP services.

- Describe UDP features.

- List out protocols that use the UDP as the transport layer protocol.

- Explain TCP services.

- Describe TCP Features.

- Explain TCP segment header format.

- Example of TCP hex dump

- Explain TCP connections establishment with diagram.

- Explain TCP data transfer with diagram.

- Explain TCP connection termination with diagram.

- List out protocols that use the TCP as the transport layer protocol.

- Draw state transition diagram of TCP.

- Explain SCTP services.

- Describe SCTP features.

- Explain SCTP packet format.

- Example of SCTP hex dump.

- Explain SCTP Association establishment with diagram.

- Explain SCTP Association termination with diagram.

- Differentiate UDP and TCP.

| UDP | TCP |
| --- | --- |
| It is a connectionless protocol. | It is a connection-oriented protocol. |
| It is unreliable protocol. | It is reliable protocol. |
| UDP datagram header size is 8 bytes. | TCP segment header size is 20-60 bytes. |
| Checksum is optional. | Checksum is mandatory. |
| There is no need to establish connection before data transfer. Hence it is faster than TCP. | There is a need to establish connection before data transfer. Hence it is slower than UDP. |
| It generates low overhead compared to TCP. | It generates more overhead compared to UDP. |
| It does not provide congestion control. | It provides congestion control. |
| An application layer protocol DHCP uses the services of UDP. | An application layer protocol HTTP uses the services of TCP. |

- Differentiate TCP and SCTP.

| TCP | SCTP |
| --- | --- |

- Differentiate UDP and SCTP.

| UDP | SCTP |
| --- | --- |
| It is a connection less protocol. | It is a connection-oriented protocol. |
| It is unreliable protocol. | It is reliable protocol. |
| UDP datagram header size is 8 bytes. | SCTP packet header size is |
| It does not provide built-in error handling. | It provides built-in error handling. |
| There is no need to establish connection before data transfer. Hence it is faster than SCTP. | There is a need to establish association before the data transfer. Hence it is slower than UDP. |
| It is datagram oriented. | It is message oriented. |
| It does not provide congestion control. | It provides congestion control. |
| An application layer protocol DHCP uses the services of UDP. | An application layer protocol HTTP uses the services of TCP. |